# Ephemerides Validation Report – V1

Project: Astronometria
Layer: Ephemerides
Status: VALIDATED (v1)
Date: 2026-02-14

## 1. Scope

Validation of the complete analytical ephemeris pipeline:

VSOP87A (heliocentric, J2000, TT)
→ Earth subtraction (geocentric, ecliptic)
→ Obliquity rotation (equatorial, J2000)
→ RA/Dec derivation

Velocity support is implemented and internally verified but not yet required in production rendering.

## 2. Reference Frame Definition

### Ecliptic Reference Frame (J2000)

- Origin: Sun (heliocentric) / Earth (after subtraction)

- X-axis: Direction of the dynamical vernal equinox (J2000)

- Z-axis: North pole of the ecliptic (J2000)

- Y-axis: Completes a right-handed system

### Equatorial Reference Frame (J2000)

- Obtained via rotation by mean obliquity ε₀

- Mean equator of J2000

- Validated against ICRF (JPL Horizons)

### Time Model

- Internal ephemeris time: TT (Julian Date TT)

- ΔT: Meeus polynomial approximation

- External validation performed using TDB (JPL Horizons)

## 3. Validation Layers

### 3.1 VSOP87A – Heliocentric Validation

Reference source: IMCCE vsop87.chk

- 10 reference epochs per planet

- Tolerance: 1e-10 AU

- All tests green

Result:
Heliocentric state vectors numerically verified.

### 3.2 Geocentric (Ecliptic) Validation

Algebraic validation:

Geo = HelioPlanet − HelioEarth

Test planets: - Mars - Venus - Jupiter

Test epochs: 1800–2000 range

Tolerance: 1e-10 AU

Result:
Earth subtraction and geocentric transformation confirmed.

### 3.3 Geocentric Equatorial (J2000) Validation

Reference source: JPL Horizons

Horizons configuration:

- Vector table

- Observer: @399 (Earth)

- Frame: ICRF

- Reference plane: equatorial-aligned inertial

- Vector correction: geometric states

- Time scale: TDB

- Units: AU-D

Test planets: - Mars - Venus - Jupiter

Test epochs: - JD 2451545.0 - JD 2415020.0 - JD 2378495.0 (where applicable)

Tolerance: 5e-7 AU

Observed deviations: Within expected VSOP vs DE ephemeris differences (tens of meters).

Result:
Independent external validation successful.

### 3.4 Quadrant & Angle Robustness

Additional validation ensured:

- RA normalization to [0°, 360°)

- Correct quadrant handling using atan2

- Stable declination sign

- No NaN or infinity in edge cases

All tests green.

### 3.5 Time Model Validation

Validated:

- Calendar → JD_TT conversion

- Direct JD_TT constructor

- Julian millennia since J2000 consistency

Tolerance: 1e-12

Result:
Time conversion deterministic and numerically stable.

## 4. Critical Debug Milestone

A temporary 90° discrepancy was observed during early Horizons comparison.

Root cause: Incorrect Horizons reference plane setting
(“body mean equator and node of date”)

Resolution: Switched to equatorial-aligned inertial frame.

Conclusion: The implementation was correct; the discrepancy originated from external configuration.

This prevented the introduction of an unnecessary coordinate rotation and preserved frame consistency.

## 5. Numerical Quality Assessment

Typical agreement with JPL DE ephemerides:

- Position differences: ~10⁻⁷ AU

- Equivalent to tens of meters

Fully consistent with analytical VSOP87A limitations.

## 6. Conclusion

The analytical ephemeris pipeline is:

- Numerically stable

- Frame-consistent

- Externally validated

- Scientifically robust

The Ephemerides layer is therefore declared:

Validated v1 – Scientifically Cross-Verified

Ready for:

- Integration into rendering

- RA/Dec projection usage

- Simulation layer expansion

End of Ephemerides Validation Report – V1